EXERCISE - 1

Homogeneity study data

| S.No of PT item | Result 1 | Result 2 |
| --- | --- | --- |
| 1 | 40.1 | 40.0 |
| 2 | 39.8 | 39.6 |
| 3 | 39.5 | 39.4 |
| 4 | 39.8 | 39.2 |
| 5 | 39.7 | 39.6 |
| 6 | 39.3 | 39.2 |
| 7 | 39.3 | 39.5 |
| 8 | 39.8 | 39.7 |
| 9 | 39.6 | 39.8 |
| 10 | 39.6 | 39.5 |

SDPA = 0.25

Calculate Between sample SD, Ss for the above data. Ss=

Stability study data

| S.No of PT item | Result 1 | Result 2 |
| --- | --- | --- |
| 1 | 38.3 | 38.5 |
| 2 | 38.8 | 38.7 |
| 3 | 38.6 | 38.8 |

Calculate the difference between homogeneity & stability average =

Determine whether the PT items or homogeneous and stable . If not what action will be taken?

EXERCISE - 2

ASSIGNMENT OF VALUE USING A CRM IN THE PT PROGRAM

CRM Value  from Reference Material Certificate =21.62

EU  from Reference Material Certificate = 0.52

Coverage factor reported in Reference Material Certificate =2

Data on characterizing each of the 20 PT items twice using the CRM are as under:

| S.No | RM1 | RM2 | CRM1 | CRM2 |
| --- | --- | --- | --- | --- |
| 1 | 20.5 | 20.5 | 19.0 | 18.0 |
| 2 | 21.1 | 20.7 | 19.8 | 19.9 |
| 3 | 21.5 | 21.5 | 21.0 | 21.0 |
| 4 | 22.3 | 21.7 | 21.0 | 20.8 |
| 5 | 22.7 | 22.3 | 20.5 | 21.0 |
| 6 | 23.6 | 22.4 | 20.3 | 20.3 |
| 7 | 20.9 | 21.2 | 21.5 | 21.8 |
| 8 | 21.4 | 21.5 | 21.9 | 21.7 |
| 9 | 23.5 | 23.5 | 21.0 | 21.0 |
| 10 | 22.3 | 22.9 | 22.0 | 21.3 |
| 11 | 23.5 | 24.1 | 20.8 | 20.6 |
| 12 | 22.5 | 23.5 | 21.0 | 22.0 |
| 13 | 22.5 | 23.5 | 21.0 | 21.0 |
| 14 | 23.4 | 22.7 | 22.0 | 22.0 |
| 15 | 24.0 | 24.2 | 22.1 | 21.5 |
| 16 | 24.5 | 24.4 | 22.3 | 22.5 |
| 17 | 24.8 | 24.7 | 22.0 | 21.9 |
| 18 | 24.7 | 25.1 | 21.9 | 21.9 |
| 19 | 24.9 | 24.4 | 22.4 | 22.6 |
| 20 | 27.2 | 27.0 | 24.5 | 23.7 |

Calculate assigned value and standard uncertainty in assigned value of the RM prepared by PTP.

Assigned Value =

Standard uncertainty in assigned value =

EXERCISE - 3

Determination of Assigned value and SDPA using ALGORITHM A

Data received from the participants are as under:

| 1 | 1.69 |
| --- | --- |
| 2 | 0.74 |
| 6 | 2.05 |
| 8 | 1.14 |
| 9 | 2.19 |
| 12 | 1.39 |
| 13 | 1.52 |
| 14 | 1.5 |
| 19 | 1.58 |
| 20 | 0.8 |
| 22 | 1.21 |
| 25 | 1.63 |

Determine the following using Algorithm A:

Assigned Value =

SDPA =

Standard Uncertainty of Assigned Value =

Also calculate Z score or Z prime score of the participants

EXERCISE - 4

Determination of Assigned value and SDPA using nIQR method

Data received from the participants are as under:

| S.No. | Value |
| --- | --- |
| 1 | 47.0 |
| 2 | 41.0 |
| 3 | 54.0 |
| 4 | 43.0 |
| 5 | 65.0 |
| 6 | 43.0 |
| 7 | 47.0 |
| 8 | 48.0 |
| 9 | 42.0 |
| 10 | 51.0 |
| 11 | 49.0 |
| 12 | 56.0 |
| 13 | 43.0 |
| 14 | 45.0 |
| 15 | 42.0 |

Determine the following using nIQR method:

Assigned Value =

nIQR =

Also calculate Z score of the participants

EXERCISE – 5 (a) @ 95% CL

Identification of Blunders using Grubbs’ test

Data received from the participants are as under:

| S.No | Value |
| --- | --- |
| 1 | 59.5 |
| 2 | 62.4 |
| 3 | 67.0 |
| 4 | 67.5 |
| 5 | 67.5 |
| 6 | 68.5 |
| 7 | 69.1 |
| 8 | 69.5 |
| 9 | 69.6 |
| 10 | 70.0 |
| 11 | 70.2 |
| 12 | 70.2 |
| 13 | 70.3 |
| 14 | 70.4 |
| 15 | 70.9 |

Use Grubbs test and identify the blunders and list them below

EXERCISE – 5 (b) @ 99% CL

Identification of Blunders using Grubbs’ test

Data received from the participants are as under:

| S.No | Value |
| --- | --- |
| LAB_01 | 54.10 |
| LAB_04 | 59.53 |
| LAB_07 | 57.37 |
| LAB_10 | 61.85 |
| LAB_12 | 51.07 |
| LAB_16 | 62.44 |
| LAB_19 | 16.56 |
| LAB_23 | 49.52 |
| LAB_26 | 55.80 |
| LAB_29 | 48.24 |

Use Grubbs test and identify the blunders and list them below

EXERCISE – 6

Identify the blunders and calculate the performance of all participants

| Code | Result |
| --- | --- |
| 2 | 314.7 |
| 5 | 317.0 |
| 9 | 327.8 |
| 10 | 313.8 |
| 17 | 335.7 |
| 19 | 330.5 |
| 21 | 328.8 |
| 24 | 302.9 |
| 27 | 385.1 |
| 28 | 339.6 |
| 30 | 342.1 |
| 32 | 320.0 |
| 33 | 322.0 |
| 37 | 338.4 |
| 39 | 248.9 |

Take MPE between as 6%

| Code nos. of blunders identified finally: |  |
| --- | --- |

| Assigned value after removal of blunders |  |
| --- | --- |
| SDPA after removal of blunders |  |
| SU after removal of blunders |  |

EXERCISE – 7 – VALIDATION OF ASSIGNED VALUE

| Example -1 Pesticide residue in Milk (mg/L) |  |
| --- | --- |
| Assigned Value= | 445.3 |
| EU of Assigned value= | 17.26 |
| Robust average of 43 participants= | 421.58 |
| Robust SD of 43 participants= | 21.47 |

| Example - 2 Aflatoxin content in ground nut (mg/kg) |  |
| --- | --- |
| Assigned Value= | 425.7 |
| EU of Assigned value= | 17.28 |
| Robust average of 19 participants= | 431.2 |
| Robust SD of 19 participants= | 14.16 |

| Example - 3 Polyester content in textiles (%) |  |
| --- | --- |
| Assigned Value from 19 participants = | 67.30 |
| SDPA= | 1.22 |
| Homogeneity average= | 66.80 |
| EU (max) of Homogeneity results= | 1.34 |

| Example - 4 Mercury content in food (mg/kg) |  |
| --- | --- |
| Assigned Value from 17 participants = | 152.5 |
| SDPA= | 6.48 |
| Homogeneity average= | 160.4 |
| EU (max) of Homogeneity results= | 5.24 |

EXERCISE – 8 STATISTICS FOR SMALL NO. OF PARTICIPANTS

Case -1: Cement fineness (Assume MPE as 30%)

| Code | Value |
| --- | --- |
| LAB-08 | 240 |
| LAB-07 | 267 |
| LAB-11 | 269 |
| LAB-13 | 270 |
| LAB-09 | 272 |
| LAB-12 | 274 |
| LAB-14 | 295 |
| LAB-05 | 300 |
| LAB-04 | 314 |
| LAB-01 | 341 |

Case -2: PR in vegetables (Assume MPE as 30%)

| Code | Value |
| --- | --- |
| EO_01 | 60.1 |
| EO_13 | 79.8 |
| EO_28 | 84.2 |
| EO_14 | 90.5 |

Calculate the following in both cases (In case 2 take SDPA by perception @10% of Assigned Value):

| Assigned Value = |  |
| --- | --- |
| SDPA = |  |
| SU of Assigned Value = |  |

EXERCISE – 9 (a)

Performance evaluation in case of qualitative PT schemes

| Assigned Value | Present | Present | Absent |  |
| --- | --- | --- | --- | --- |
| Data received from the participants are as under: |  |  |  |  |
| Lab code | Vial A | Vial B | Vial C | Performance (Overall) |
| 1 | Present | Absent | Present |  |
| 2 | Present | Present | Absent |  |
| 3 | Present | Present | Absent |  |
| 4 | Present | Present | Absent |  |
| 5 | Absent | Absent | Absent |  |
| 6 | Present | Present | Absent |  |
| 7 | Present | Present | Absent |  |
| 8 | Present | Present | Absent |  |
| 9 | Present | Present | Present |  |
| 10 | Present | Absent | Present |  |
| 11 | Present | Present | Absent |  |
| 12 | Present | Present | Absent |  |
| 13 | Present | Present | Absent |  |
| 14 | Present | Present | Absent |  |
| 15 | Present | Present | Absent |  |

Give the overall performance of each of the participants in the above PT Scheme

EXERCISE – 9 (b)

Performance evaluation in case of semi quantitative PT schemes

| Data received from the participants are as under: |  |  |  |  |
| --- | --- | --- | --- | --- |

| ID of Lab | Rating reported | Performance of the lab |
| --- | --- | --- |
| 1 | 2.5 |  |
| 2 | 3 |  |
| 3 | 2.5 |  |
| 4 | 2.5 |  |
| 5 | 1.5 |  |
| 6 | 3 |  |
| 7 | 3.5 |  |
| 8 | 3 |  |
| 9 | 3 |  |
| 10 | 2.5 |  |
| 11 | 3 |  |
| 12 | 3.5 |  |
| 13 | 3 |  |
| 14 | 3 |  |
| 15 | 2.5 |  |
| 16 | 2 |  |

Determine Assigned value and evaluate the performance of all the labs